Instructions for Transtibial Amputees Using Silicone Liner and Pin

Locking Suspension System

Application:

- Remove any compression socks or protector

- Roll the silicone liner inside out and flatten the end before placing it on your limb.  There should be no gapping between the liner and your limb.

- Continue to roll the liner onto your limb being careful of the pin (the pin can poke the liner if you are not careful, if this happens, contact your prosthetist).

- The liner should be rolled completely onto the limb and never pulled onto the limb.  Tension on the skin may cause skin irritation or blisters. Rolling the top 1/3 of the liner up and down a couple of times will help reduce the tension.

- Once the liner is in place continue to add the appropriate amount of prosthetic sock ply.

- Be sure that the socks are free of wrinkles

- Maintaining the correct amount of prosthetic sock ply throughout the day will be an important part of a successful prosthetic experience.

- If at any time your prosthesis seems too loose or too tight, you will need to modify the amount of sock ply.

- It is recommended that you carry an assortment of sock ply with you for changes in volume (size of your limb) throughout the day.

- If you have a pelite liner, place that on your limb next, if not continue with next step.

- Step into your prosthesis until you are securely locked in.

Removing the Prosthesis:

- Push in and hold the button to release the lock mechanism.  If the lock sticks you may have to put weight onto your prosthesis and then disengage the lock.

- Remove your limb from the prosthesis.

- Remove the liner if applicable

- Remove any prosthetic socks and then roll your liner off your limb.

If at any time you have questions or concerns, please contact your prosthetist.